TABLE OF CONTENTS

pccard.library/PCCard_FreeTupleInfo

pccard.library/PCCard_GetTupleInfo

pccard.library/PCCard_FreeTupleInfo       pccard.library/PCCard_FreeTupleInfo

NAME

PCCard_FreeTupleInfo -- Relinquish a CIS tuple tag list.

SYNOPSIS

PCCard_FreeTupleInfo(tag_list)

A0

VOID PCCard_FreeTupleInfo(struct TagItem *);

FUNCTION

Frees a tag list obtained via PCCard_GetTupleInfo().

INPUTS

tag_list - Tuple tag list (may be NULL).

RESULT

None.

EXAMPLE

NOTES

BUGS

SEE ALSO

PCCard_GetTupleInfo()

pccard.library/PCCard_GetTupleInfo         pccard.library/PCCard_GetTupleInfo

NAME

PCCard_GetTupleInfo -- Parse a CIS tuple.

SYNOPSIS

tag_list = PCCard_GetTupleInfo(tuple)

D0                             A0

struct TagItem *PCCard_GetTupleInfo(UBYTE *);

FUNCTION

Reads the information held within a CIS tuple and presents it in the

form of a tag list. The returned tag list is read-only and must be

relinquished via PCCard_FreeTupleInfo().

The tags available for each tuple type are described below.

PCCARD_TPL_DEVICE/PCCARD_TPL_DEVICEA:

PCCARD_RegionCount - The number of memory regions in either

common memory (PCCARD_TPL_DEVICE) or attribute memory

(PCCARD_TPL_DEVICEA).

PCCARD_RegionLists - An array of tag lists, one for each memory

region. Each sub-list may have the following tags:

PCCARD_Flags - Currently only PCCARD_REGIONF_WP (write

protection) is defined.

PCCARD_Type - Region's memory type.

PCCARD_Speed - Speed in nanoseconds.

PCCARD_Base - Offset of start of region.

PCCARD_Length - Length of region.

PCCARD_TPL_VERS1:

PCCARD_MajorVersion - ?

PCCARD_MinorVersion - ?

PCCARD_InfoStringCount - Number of strings available.

PCCARD_InfoStrings - An array of card information strings.

PCCARD_TPL_CONFIG:

PCCARD_RegisterBase - Offset within attribute memory of

configuration registers.

PCCARD_ModeCount - Number of operating modes.

PCCARD_TPL_CFTABLEENTRY:

PCCARD_ModeNo - Value to write to the COR to select this mode.

PCCARD_Flags - See pccard.h for details.

PCCARD_VCCPowerTags - Tag list for VCC power. The following tags

may be present (voltages in 10-uV units, currents in 100-nA

units):

PCCARD_NominalVoltage - Nominal supply voltage.

PCCARD_MinVoltage - Maximum supply voltage.

PCCARD_MaxVoltage - Minimum supply voltage.

PCCARD_StaticCurrent - Continuous supply current required.

PCCARD_AverageCurrent - Maximum current averaged over one

second.

PCCARD_PeakCurrent - Maximum current averaged over 10 ms.

PCCARD_DownCurrent - Current required in power-down mode.

PCCARD_Flags - See pccard.h for details.

PCCARD_VPP1PowerTags - Tag list for VPP1 power. Uses the same

tags as PCCARD_VCCPowerTags.

PCCARD_VPP2PowerTags - Tag list for VPP2 power. Uses the same

tags as PCCARD_VCCPowerTags.

PCCARD_WaitTimingTags - Tag list for wait timing. The following

tags may be present:

PCCARD_Value - Base value/mantissa (in nanoseconds).

PCCARD_Scale - Multiplier/exponent to be applied to

PCCARD_Value.

PCCARD_ReadyTimingTags - Tag list for ready timing. Uses the

same tags as PCCARD_WaitTimingTags.

PCCARD_ReservedTimingTags - Tag list for reserved timing. Uses

the same tags as PCCARD_WaitTimingTags.

PCCARD_IOFlags - See pccard.h for details.

PCCARD_IOLineCount - Number of IO lines decoded by the card.

PCCARD_IOWinCount - Number of IO windows.

PCCARD_IOWinBases - An array of ULONGs giving the window bases

as offsets into the card's IO space.

PCCARD_IOWinLengths - An array of ULONGs giving the lengths of

the IO windows.

PCCARD_IRQFlags - See pccard.h for details.

PCCARD_IRQMask - Interrupt numbers that may be used.

PCCARD_MemWinCount - Number of memory windows.

PCCARD_MemWinBases - An array of ULONGs giving the window bases

as offsets into the card's memory space.

PCCARD_MemWinHostBases - An array of ULONGs giving the window

bases as offsets into the host memory space.

PCCARD_MemWinLengths - An array of ULONGs giving the lengths of

the memory windows.

PCCARD_TPL_MANFID:

PCCARD_Maker - The card's manufacturer ID.

PCCARD_Product - The card's product ID relative to its

manufacturer.

PCCARD_TPL_FUNCID:

PCCARD_Type - The card's function, eg. network or memory.

PCCARD_Flags - Card initialisation flags.

INPUTS

tuple - Pointer to the tuple.

RESULT

tag_list - A tag list containing the tuple's properties.

EXAMPLE

NOTES

BUGS

SEE ALSO

PCCard_FreeTupleInfo()